STABILITY ANALYSIS OF COLLOIDAL DISPERSIONS

Hélène Dihang1 & Eran Golik2

1Formulaction, L’Union (Toulouse), France
2Dr. Golik Chemical Instrumentation, Tel Aviv, Israel

E-mail: golik@golik.co.il

Applications of colloidal dispersions in the industry are numerous and concern practically every field (cosmetics, food, ink, polymers, lubricants, etc.). Therefore, in the last decades a vast amount of scientific research has been developed to improve the understanding of complex dispersions. Using a scientific approach and new analytical techniques, it is now possible to control and tailor properties of  emulsions, suspensions and foams and to get a better understanding of their time behaviour. Following this idea, stability measurements, which were commonly done by simple visual observations, can now be performed automatically via an optical device.

The TurbiscanÒ is based on Multiple Light Scattering (MLS) and is associated to a vertical scanning of the sample. It is like a high resolution electronic eye, enabling to identify and quantify instability phenomena before they are visible to the operator (5 to 50 times earlier than the naked eye). Physical parameters and kinetics can be computed in order to facilitate and improve comparisons between formulations. The system is suitable for both diluted or concentrated products (0 to 90% v/v). Moreover, it is possible to use the information contained in the MLS signal to extract parameters such as particle mean diameter without the need of dilution, as often required with diffraction techniques based on single scattering models. This measurement can be done on the finished product or, in real time, during the process.